# Design and Implementation of FIR and IIR Filters to Clean Noisy Signals Using Python

PYTHON CRT PROJECT EEE-B/SECTION

Team members

- 23701A0248

- 23701A0254

- 23701A0261

- 23701A0276

- 23701A0278

- 23701A0288

- 24705A0215

- 24705A0217

- 24705A0218

- 24705A0219

- 24705A0222

- 24705A0224

- 24705A0226

- 24705A0229

- 24705A0231

- 24705A0235

Table of Contents

  - Introduction

  - Fundamentals of Digital Filters

  - FIR Filters: Theory and Design

  - IIR Filters: Theory and Design

  - Problem Statement and Objectives

  - Software and Libraries Used

  - Overview of the Python Program

  - Detailed Explanation of Code

  - Signal Generation and Noise Model

  - FIR Filter Design and Implementation

  - IIR Filter Design and Implementation

  - Applying Filters to Noisy Signal

  - Frequency Response Analysis

  - Visualization of Results

  - Performance Comparison Between FIR and IIR

  - Practical Applications

  - Extending the Project

  - Challenges and Solutions

  - Conclusion

### Introduction

Background:-

In the field of digital signal processing (DSP), filters are used extensively to manipulate signals to enhance useful information and reduce noise or unwanted components. Many real-world signals—whether audio, biomedical, or sensor outputs—contain noise which can obscure important characteristics. Filtering is essential to improving signal quality for analysis or further processing.

### Objective:-

This project focuses on two primary digital filter types: FIR (Finite Impulse Response) and IIR (Infinite Impulse Response). Using Python, SciPy, and Matplotlib, the goal is to design, implement, and compare these filters to clean noisy signals.

### Structure:-

  - Introduce theory behind FIR and IIR filters.

  - Design filters using Python libraries.

  - Generate noisy test signals.

  - Apply filters and visualize results.

  - Compare filter performance.

## Fundamentals of Digital Filters

### Discrete-Time Signals:-

Signals processed in digital systems are discrete in time, sampled at a sampling frequency fs. The Nyquist frequency fN=fs/2defines the maximum frequency representable without aliasing.

### Digital Filters Overview:-

Filters process discrete samples to modify frequency components:

  - Lowpass filters allow frequencies below cutoff fc.

  - Highpass filters allow frequencies above fc.

  - Bandpass filters allow frequencies between two cutoffs.

  - Bandstop filters attenuate frequencies between two cutoffs.

### Impulse Response and System Function:-

A filter’s impulse response defines how it responds to a single impulse. The system function H(z) characterizes filter behavior in the z-domain.

## FIR Filters: Theory

### FIR Filter Definition

An FIR filter has a finite number of coefficients bk:

where N is the order of the filter.

### Characteristics:-

  - Non-recursive: output depends only on input.

  - Always stable.

  - Can have exactly linear phase.

  - Typically higher order needed for sharp transitions.

### Design Using Window Method:-

The ideal lowpass filter impulse response is truncated using a window function (e.g., Hamming window) to limit length and reduce sidelobes.

4: IIR Filters: Theory

# IIR Filter Definition:-

An IIR filter uses both inputs and previous outputs:

### Characteristics:-

  - Recursive filter; output feedback.

  - Can achieve sharper cutoff with fewer coefficients.

  - Stability depends on pole locations.

  - Phase response is typically nonlinear.

### Butterworth Filter:-

A Butterworth filter is maximally flat in the passband and has monotonic magnitude response, commonly used for general- purpose lowpass filtering.

## 5: Problem Statement and Objectives

#### Problem Statement

Noisy signals complicate analysis and reduce system performance. The goal is to remove noise effectively without distorting signal features.

#### Objectives

  - Generate noisy test signals with known frequencies.

  - Design FIR and IIR lowpass filters using Python.

  - Apply filters to noisy signals.

  - Plot signals and filter frequency responses.

  - Compare filter performance.

## 6: Software and Libraries

### Python 3.x:-

The project is implemented in Python for its extensive scientific ecosystem.

### Libraries:-

  - NumPy: Efficient numerical computations and array handling.

  - SciPy.signal: Signal processing functions including filter design and application.

  - Matplotlib: Visualization of time series and frequency responses.

Installation:-

pip install numpy scipy matplotlib

## 7: Signal Generation and Noise Model

### Clean Signal:-

We create a clean sine wave:

x(t)= sin(2πft)

with frequency f=5Hz.

### Noise Model:-

Additive Gaussian noise and sinusoidal interference at 50 Hz are added

where noise is white Gaussian noise.

## 8: FIR Filter Design in Python

### Design Parameters:-

  - Filter order

  - Cutoff frequency fc=10 Hz.

  - Sampling frequency fs=500 Hz.

### Design Code:-

from scipy.signal import firwin

nyq = 0.5 * fs

normalized_cutoff = cutoff / nyq

fir_coeff = firwin(numtaps=order+1, cutoff=normalized_cutoff, pass_zero='lowpass')

### Explanation:-

  - numtaps is filter length = order + 1.

  - pass_zero='lowpass' designs a lowpass filter.

## 9: IIR Filter Design in Python

### Butterworth Design:-

from scipy.signal import butter

b, a = butter(N=order, Wn=normalized_cutoff, btype='low')

  - N is filter order.

  - Wn is normalized cutoff.

  - btype='low' for lowpass.

## 10: Applying Filters

### FIR Filtering:-

from scipy.signal import lfilter

filtered_fir = lfilter(fir_coeff, 1.0, noisy_signal)

### IIR Filtering:-

Use zero-phase filtering to avoid phase distortion:

from scipy.signal import filtfilt filtered_iir = filtfilt(b, a, noisy_signal)

11: Frequency Response

### FIR Filter:-

from scipy.signal import freqz

w, h = freqz(fir_coeff) plt.plot(w * fs / (2*np.pi), 20 * np.log10(abs(h)))

#### IIR Filter:-

Same method with numerator and denominator coefficients.

## 12: Visualization of Signals

noisy signal,FIR filtered and IIR filtered signals in time domain to visually assess noise reduction.

## 13: Performance Comparison

Goal:-

Effectively suppress undesired components such as high- frequency noise (e.g., 50 Hz interference) while preserving the desired signal (5 Hz sine wave).

### FIR Filter:

- Strengths: Offers strong attenuation of frequencies above the cutoff, especially with higher filter orders.

- Observed Performance: The FIR filter successfully removed high-frequency noise, showing a smooth waveform in the filtered signal.

- Limitation: Requires a higher order to achieve a sharp cutoff, especially for narrow transition bands.

# IIR Filter:

- Strengths: Achieves sharp roll-off near the cutoff frequency with relatively lower order.

- Observed Performance: The IIR filter effectively eliminated the 50 Hz interference with fewer coefficients.

- Trade-off: More susceptible to ripple or gain irregularities near the cutoff band.

Verdict: Both filters attenuate noise well; IIR does it more efficiently (lower order), but FIR does so more predictably.

## 14: Applications

Filtering is vital in:-

- Audio denoising.

- Biomedical signal cleanup (ECG, EEG).

- Communications.

- Sensor data processing.

## 15: Extending the Project

Ideas include:

- Designing bandpass and highpass filters.

- Adaptive filtering techniques.

- Real-time filtering.

- Multirate filtering.

## 16: Challenges

- Choosing filter parameters.

- Managing phase distortion.

- Balancing order vs complexity.

- Noise modeling accuracy.

## 17: Full Python Code Listing

import numpy as np

import matplotlib.pyplot as plt

from scipy.signal import firwin, lfilter, freqz, butter, filtfilt

def design_fir_filter(order, cutoff, fs, filter_type='low'): nyq = 0.5 * fs

normalized_cutoff = np.array(cutoff) / nyq

if filter_type == 'low':

pass_zero = True

elif filter_type == 'high':

pass_zero = False

elif filter_type in ['bandpass', 'bandstop']:

pass_zero = filter_type else:

raise ValueError("Invalid filter_type. Choose from 'low', 'high', 'bandpass', 'bandstop'.")

fir_coeff = firwin(order + 1, normalized_cutoff, pass_zero=pass_zero) return fir_coeff

def design_iir_filter(order, cutoff, fs, filter_type='low'): nyq = 0.5 * fs

normalized_cutoff = np.array(cutoff) / nyq

b, a = butter(order, normalized_cutoff, btype=filter_type, analog=False) return b, a

def apply_filter(b, a, signal, use_filtfilt=False): if use_filtfilt:

filtered_signal = filtfilt(b, a, signal) # zero-phase filtering else:

filtered_signal = lfilter(b, a, signal) return filtered_signal

def plot_frequency_response(b, a=1, fs=1.0, title='Frequency Response'): w, h = freqz(b, a, worN=8000)

plt.plot((fs * 0.5 / np.pi) * w, 20 * np.log10(abs(h)), 'b') plt.title(title)

plt.xlabel('Frequency (Hz)') plt.ylabel('Gain (dB)') plt.grid(True)

plt.show()

if   name	== "  main  ":

fs = 500.0	# Sampling frequency (Hz)

t = np.arange(0, 2.0, 1/fs) # Time vector for 2 seconds

# Create a noisy signal: 5 Hz sine + white noise + 50 Hz interference freq_signal = 5.0

noisy_signal = (

np.sin(2 * np.pi * freq_signal * t) +

0.5 * np.random.randn(len(t)) +

0.3 * np.sin(2 * np.pi * 50 * t)

)

# Filter parameters fir_order = 50

iir_order = 4

cutoff = 10 # cutoff frequency (Hz) filter_type = 'low' # lowpass filter

# Design FIR filter

fir_b = design_fir_filter(fir_order, cutoff, fs, filter_type)

# Design IIR filter

iir_b, iir_a = design_iir_filter(iir_order, cutoff, fs, filter_type)

# Apply filters

fir_filtered = lfilter(fir_b, [1.0], noisy_signal)

iir_filtered = apply_filter(iir_b, iir_a, noisy_signal, use_filtfilt=True)

# Plot time domain signals plt.figure(figsize=(12, 8))

plt.subplot(3, 1, 1)

plt.plot(t, noisy_signal, label='Noisy Signal') plt.legend()

plt.xlabel('Time [s]') plt.ylabel('Amplitude')

plt.subplot(3, 1, 2)

plt.plot(t, fir_filtered, label='FIR Filtered Signal', color='orange') plt.legend()

plt.xlabel('Time [s]') plt.ylabel('Amplitude')

plt.subplot(3, 1, 3)

plt.plot(t, iir_filtered, label='IIR Filtered Signal', color='green') plt.legend()

plt.xlabel('Time [s]') plt.ylabel('Amplitude')

plt.tight_layout()

plt.show()

# Plot frequency responses

plot_frequency_response(fir_b, fs=fs, title='FIR Filter Frequency Response') plot_frequency_response(iir_b, iir_a, fs=fs, title='IIR Filter Frequency Response'

## 18: Experimental Results

- Filter effectiveness.

- Frequency response plots.

- Signal waveforms.

## 19: Conclusions

Summary of findings: FIR filters provide stable linear phase response; IIR filters are efficient but phase nonlinear. Both can effectively clean noisy signals when properly designed.